Step-by-step procedure to detect and remove pacing artifacts from ECG signal:

- Download ECGApp_final.m, kors.m, Test_file1.mat, Test_file2.mat.

- Open the file (ECGApp_final.m) in Matlab.

- Run

- Click on ‘Load ECG’ to load the test file.

- Kors-transformed orthogonal leads X, Y, Z and 12-leads will be displayed in separate figures.

- Click on ‘Process’

- PC1 will be displayed with detected peaks (red circled) and locations (in sample) using default thresholds.

  - If thresholds need to be adjusted, turn on the Custom Threshold switch.

  - Put custom height and distance threshold and click Process to ensure all spikes in PC1 are detected.

- If atrial artifacts are present and cannot be detected (in PC1 figure) using the above method (5 and 6), click on ‘Step 2’.

- Check PC1 plot again.

- If artifacts are still undetected (or in case of misdetection), click on ‘Add’ or ‘Remove’. To remove a mis-detected location, select the location from ‘Peak Index’ list and click on 'Remove’. To add an undetected location, click on ‘Add’ and provide the location (in sample) to the popped-up box.

- After all the peaks are selected, click on ‘Plot Results’.

- A new window will pop up with two separate lists of leads. One list belongs to the leads with artifacts and another without. Users can override algorithm decision by checking/unchecking boxes beside each lead. Click on ‘Apply Correction’.

- Corrected (after removing pacing artifacts) 12-leads and orthogonal X, Y, Z leads will be displayed in the figure on top of the leads with pacing artifacts.

- To change the artifact width, enable the switch under ‘%Envelope/Sample’, to increase the width enter values below 0.3, to decrease the width enter values above 0.3 (default 0.3), alternatively, switch the method to Sample and enter the amount of data to remove, this value will be removed both before and after the artifact peak.

- Click on ‘Reviewed’ to move the processed file to ‘Reviewed’ folder.

- If an error occurs, click on ‘Error’ to move the file to ‘Error’ folder and review the file later.

- If user decides that there are no pacing artifacts in the signal, click on ‘No transformation’ to move the file to ‘No change’ folder.